QO-100 WB Spectrum Ryde Tuner

User & Operator Manual

User & Operator Manual

Live QO-100 wideband spectrum viewing and receiver channel selection

Version 1.1 — September 2026

System Overview

The QO-100 wideband transponder is an approximately 8–9 MHz linear amateur-radio repeater payload aboard

Es'hail-2 (Qatar-OSCAR 100), used primarily for digital amateur television (DATV) and other wideband

transmissions.

| Parameter | Value |
| --- | --- |
| Orbit | Geostationary at 25.9° East |
| Uplink | 2401.5–2409.5 MHz |
| Downlink | 10491.0–10499.0 MHz |
| Bandwidth | Approximately 8–9 MHz |
| Polarisation | Horizontally polarised |

Foot print of the QO-100 Satellite show the surface area of the world covered by the satellite

Live Viewing

| SRT Stream | srt://g8koe.ddns.net:8890?streamid=read:qo100 |
| --- | --- |
| Web Chrome Browser | http://g8koe.ddns.net:8888/qo100/ |
| Web Browser Tune Control | http://webtune.lebs.org.uk:5000 |

Receiver Control Web Interface

To be able to control the tuning of the satellite receiver open a web page and navigate to http://webtune.lebs.org.uk:5000/

Receiver-control page showing the live spectrum, frequency field, symbol-rate selector and Tune Ryde control.

Connecting to the QO-100 SRT Channel

Use the following sequence:

- Open your chosen SRT-capable viewing software.

Recommended software is the Haivison Play ISR Video Player software, Link to 	software below.

https://www.haivision.com/products/download-play-isr

- Open web control page.

http://webtune.lebs.org.uk:5000/

3. Confirm that DATV video is being received.

Example DATV video received from the QO-100 SRT stream.

If no transmission is visible, open the receiver-control web interface and check the spectrum and available

channels.

Selecting and Tuning a DATV Channel

The spectrum display represents the wideband transponder and allows a transmission to be selected directly.

The normal operating sequence is:

1. Locate the required DATV signal on the spectrum.

2. Move the mouse pointer to the centre of the signal.

3. Click the centre of the transmission.

4. Symbol rate selection should be automatic when the required signal to view has been selected by clicking on the spectrum display, but you have the option from the drop down menu to choose a symbol rate.

5. Click the blue Tune Ryde button.

The receiver should tune to the selected channel and the transmission should appear in the SRT player or

browser window.

Symbol Rate Selection

The selected symbol rate must match the transmission being tuned. The supplied operating example specifically

uses 500 KS. If the symbol rate does not match the transmission, the Ryde receiver will not correctly decode or

display the DATV service.

Beacon Failure Condition

Note that the QO-100 beacon can occasionally fail. When this occurs, receive picture

may no longer look normal, we just have to wait until the beacon resets its self-back to normal.

Example of the abnormal DATV display

Troubleshooting Qatar-OSCAR 100

Receiver does not tune

Confirm a signal is selected, the correct symbol rate is chosen and Tune Ryde is pressed.

Picture is not decoded

Verify that the selected symbol rate matches the transmission.

Control page will not open

Confirm the actual receiver http://webtune.lebs.org and port 5000.are selected

Please note that depending on the format of the transmission from the originating station not all transmissions are receivable.

Only one QO-100 SRT channel available, if more than one operator selects a channel using the WebTune application we will end up with the channel constantly changing. I will at how best to resolve this issue when time allows so please play fair when using this system.

Support and Feedback

For general comments, suggestions or offers of help email gu6efb@lebs.org.uk

Most importantly have fun and enjoy the hobby.

Thanks to the Qatar-OSCAR 100, BATC, Amsat DL, G8KOE for use of the SRT Repeater and Rob M0DTS for the inspiration from his Quicktune software

https://batc.org.uk/